Convert cassette tapes to .WAV files

Equipment: Tape2PC Converter USB cord Laptop and power cord “Y” headphone cable Mouse

The quality of the recording is not guaranteed. Please remember that cassette tapes degrade over time, and the quality of the original cassette tape was not the quality that we expect today. Copying is done in real time – a one hour cassette tape will take one hour to copy. Make sure you will finish before the library closes!

Plug in the Laptop Plug in the Tape2PC converter

Plug USB cable into laptop and the back of the Tape2PC converter (the connector will look like it isn’t plugged in all the way at the Tape2PC converter)

You can listen while you copy by plugging in the red and black ends of the “Y” cable into the Play Output connectors on the back of the Tape2PC converter. Plug your headphones into the other end of the “Y” cable. We have earbuds for sale at the circulation desk if you need them. Headphones

Turn on the laptop Click on the patron user’s icon and the type the password patron

Turn on Tape2PC converter Power button

Open (Stop) Open the A deck of the Tape2PC converter

Load your cassette tape into the A deck. The tape should be on the bottom of the cassette. The side that you will record will be facing you, and the tape should be on the left reel when the cassette is cued at the beginning. If the door doesn’t close, take the tape out and put it back in. Don’t push the tape in too far.

Double Click EZ Vinyl-Tape Converter icon on the laptop

Click Next

Click Record on the laptop and firmly press the play button on the A deck at the same time. The red dot in the record button will turn into green arrows. The play button will click and stay pushed in. Play.

When you have finished recording: Click Next on the laptop to stop the recording, push the stop button on the tape player, too. If you record the entire side of a cassette, it will record each track in an individual file. when the cassette ends, the recording will end by itself.

Name the Artist, Album, and Track title You can change the save location by clicking on the folder icon. If you do not enter this information, the files will be saved with generic names (i.e. Track 1.wav). Click Next to save the files Folder icon.

Continue recording, OR See what you have saved by Opening the Save Location, OR or exit the software by Closing the EZ Vinyl/Tape Converter

If you saved your files on the laptop, please transfer them to a USB drive (you can purchase one at the library), or to cloud storage. The default location for the files on the computer is the Music folder Please delete any files from our laptop. Make sure to take your cassette. Turn the laptop off. Turn the Tape2PC converter off.

If you want to convert your .wav tapes to .mp3s, you can use Audio Converter by MediaHuman at http://www.mediahuman.com/audio-converter/ .mp3 files are much smaller than .wav files, but have less fidelity.