Scan Photographs, Slides and Negatives

| Equipment: Epson V600 scanner and power cord USB cord Laptop and power cord Mouse (optional) |  |  |
| --- | --- | --- |
| Set up |  |  |
| Plug in the scanner and the laptop Grey Thingy Move the grey locking thingy on the back of the scanner towards the Hole for power cord center of the scanner |  |  |
| Plug one end of the USB cable into the back of the scanner, and the other end into the laptop |  |  |
| Turn on the laptop. On button Turn on the scanner with the button on the right side of the scanner The scanner will blink and beep after the laptop has booted up. |  |  |
| On the laptop, click on the patron user’s icon and the type the password patron |  |  |
| The Basics There are separate instructions for placing your originals for: Photographs Slides Negatives (35mm) Negatives (medium format) Instructions for adjusting scanning details are independent of your image’s format. |  |  |
| Photographs | Photographs: Place your photograph face down in the upper right corner next to the arrow You may scan more than one photo at a time, just leave 20 mm (0.8”) between photos Jump to Scanning Instructions on Page 6 |

| Negatives (35mm) | Negatives (35mm): Position the film holder with the narrow slots on the right. Open the cover of the 35mm film strip holder |  |
| --- | --- | --- |
|  | Hold the edges of the film Place one or two film strips into the film holder with the shiny side facing down. The sky of the picture should be at the top of the holder. Slide the top of each strip under the clips Your images should appear backwards |  |
|  | Put the small tabs on the top of the cover into the film holder first. Replace the cover and press the edges until you hear it click all the way around |  |
|  | Place the film holder on the scanner glass lining up the “A” tab on the holder with the “A” tab to the left of the glass Jump to Scanning Instructions on Page 6 |  |
| Slides | Slides: Place the film holder on the scanner glass lining the “C” tab on the side of the holder on top of the “C” location at the left of the glass |
|  | Place up to 4 slides in the film Holder with the shiny side down. Your images should appear backwards. The ground of the picture should be Down or on the right hand side Jump to Scanning Instructions on Page 6 |

| Negatives (Medium Format - uncommon) |  | Negatives (Medium Format - Uncommon): Ask for the film holder at the desk. Open the cover of the medium format film holder Hold the film by the edges Slide one medium format film image into the film holder with the shiny side facing down. Your images should appear backwards. |
| --- | --- | --- |
|  |  | Replace the cover and press the edges until you hear it click all the way around |
|  |  | If the film is curled, use the Scanning Support sheet. Leave 2mm space between the image frame and the support sheet. Curled Downwards Place the support sheet glossy side up under the film; Curled Upwards Place the support sheet glossy side down over the film. |
|  |  | Place the film holder on the scanner glass lining up the “B” tab on the holder with the “B” tab to the left of the glass Jump to Scanning Instructions on Page 6 |
| Scanning your original (all formats) |  |  |
| Document Mat |  | For photographs, the document mat should be installed For negatives and slides, the document mat should be removed. Photographs Make sure that the document mat is installed in the scanner cover Document Mat; Negatives and Slides Remove the document mat by gently sliding it up 4 clips hold the document mat. |
| Close the scanner cover gently so that your originals do not move |  |  |
| Double click the Epson Scan icon on the laptop |  |  |
| We recommend Home Mode or Professional Mode Home Mode provides some Image Adjustments and allows you to scan Click on the down arrow to see these choices more than one photograph at a time. Professional Mode provides more specific Image Adjustments, but allows only one photograph at a time. Both modes allow multiple slides and negatives. |  |  |
| Choose the appropriate document type. Note that this looks different in Home Mode and Professional Mode Home Mode Photographs – Photographs Click on the down arrow to see these choices Slides – Positive Film Negatives – Negative Film If you are in Home Mode, go to page 9 Professional Mode Photographs – Reflective Photo; Negatives – Film Color or B&W Negative Film Slides – Film Positive Film. |
| Next you need to decide how you will use the files you create of your pictures: If you will ever want to print your photos, use 300 DPI If you only want to view your photos on a computer, use 96 DPI The Resolution is expressed in Dots Per Inch (DPI). A higher DPI means a more detailed image. Images that will be uploaded to the web are usually 96 DPI or 72 DPI. Images that will be printed are usually 300 DPI. If you will want to enlarge the image, you need to use more than 300 DPI – so that when you are finished enlarging it the resulting image will have 300 DPI. Home Mode Choose the destination to choose the resolution Screen/Web = 96 DPI Printer = 300 DPI Other allows you to choose a different value for your resolution; Professional Mode Choose the resolution on the main screen.. |
| Optional |  | Image Adjustments help you to bring the best out in your photographs. Four adjustments are available in both Home mode and in Professional Mode: Descreening Removes the rippled pattern that can appear in subtly-shaded image areas, such as in skin tones. Also improves results when scanning magazine or newspaper images which include screening in their original print processes. Color Restoration - Restores the colors in faded photos automatically. Backlight Correction - Removes shadows from photos that have too much background light. DIGITAL ICE Technology - Remove dust marks that the Dust Removal feature cannot. DIGITAL ICE Technology takes longer to scan and also uses more of your system resources. |
| Optional | Home Mode has a Brightness button which allows adjustment of both brightness and contrast |  |  |
|  | Professional Mode adds the ability to tune those adjustments as well as two new adjustment options: Unsharp Mask - Turn on to make the edges of image areas clearer for an overall sharper image. The level of sharpness is adjustable. Turn off to leave softer edges. Dust Removal - Removes dust marks from your originals automatically. The results of Dust Removal do not appear in the preview, only in your scanned image. The level of dust removal is adjustable. Additionally, five icons are available for more detailed adjustments: |  |  |
| Click the Preview button to see the image that will be saved Home Mode; Professional Mode. |
| Optional | If you want to crop the image you can hover your mouse over the image. When the cursor becomes a large Plus Sign (+) click and drag to choose the area you wish to scan. If your cursor does not become a large Plus Sign, try clicking the Marquee tool button to cancel a previous selection. Cancel previous Marquee selection |  |  |
| Choose Scan to perform the scan. Home Mode; Professional Mode. |
| These are the default values: The File Save Setting box lets you choose: Location – where will your files be saved File name – Typically, you choose a prefix for The file name and consecutive numbers are Assigned to each image. For example, if your Prefix is “img” and your start Number is 040 Then the next images will be called img040, Img041, img042 etc. Image format – JPG, Bitmap, TIFF, PDF When you click the OK box, the scan is run. |  |  |  |
| When you have finished scanning, copy your files to a thumb drive or save them in Google Docs, Dropbox or another cloud location. Thumb drives are available for purchase at the Circulation Desk for $10. The default location for the pictures is the Pictures folder. You can select all of the pictures with Control-a. Then click and drag them all at once. Please delete your pictures from our laptop. Please choose Configuration and reset the setting to their default values on the “Other” tab of the Configuration box by clicking the “Reset All” button. Other Reset All Configuration Please replace the Document Mat white side out if you scanned either negatives or slides |  |  |  |
| Turn off the laptop Turn off the scanner on the right hand side Please unplug the USB cable and then move the grey thingy on the back of the scanner into the locked position by sliding towards the side of the scanner before you move the scanner. |  |  |  |